# Overview

This paper describes the composition of a linear editing station, made up of primarily Sony equipment, in the studio of the Broadcast Television Engineering Technology program at Napa Valley College.  The intent of this paper is to document the specific components comprising the system and the interconnections among the components, in case the system needs to be reconstructed.  This system is referred to in this paper as the Sony Linear Editing Station, or SLES.

# System Component

This is the list of components comprising the SLES, including the following:

- code – the code used to refer to the component in tables later in the document

- description – in the best case, the make and model number of the component, and in the worst case, a description of the component

| Code | Description |
| --- | --- |
| A | Sony 8-channel Audio Mixer MXP-29 |
| B | Electrodyne equalizer |
| C | Sony Automatic Editing Control Unit BVC-900 |
| D | Conrac SNA 14/C 13” monitor |
| E | Sony PVM-1271Q 12” monitor |
| F | Sony PVM-8220 7 ½” monitor, marked 1 and #3 |
| G | Sony PVM-8220 7 ½” monitor, marked 2 and #2 |
| H | Sony PVM-8220 7 ½” monitor, marked Rec and #1 |
| I | Sony Microcomputer SMC-210DL6 |
| J | Sony CPD-121 Character Display (12” monitor) |
| K | Sony keyboard control for the BVE-900 |
| L | Sony PVM-8220 7 ½” monitor, marked #4 |
| M | Sony PVM-122 12” monitor |
| N | Sony Betacam Videocassette Player BVW-10, marked #1 |
| O | Sony Betacam Videocassette Player BVW-10, marked #2 |
| P | Sony Betacam Videocassette Recorder BVW-40 |
| Q | Sony Special Effects Generator SEG-2550 |
| R | Sony APM-007AV/P 4 ohm shielded 25W max 12.5W nominal mini computer speakers (pair) |

Table 1: System Components

# Component Connections

This section of this document contains lists of the connections that exist between each component, for use by someone re-assembling the system.  This section describes each connection on each component, including:

- the component code

- the name of the port

- the type of connector used by the wire attached to the port

- the type and color of the wire attached to the port

- the code and port on the other side of the connecting wire

## Component A – Audio Mixer

| Port Name | Connector and Wire | Destination (Code, Port) |
| --- | --- | --- |
| Line out R | XLR | P – Audio Input Channel 2 |
| Line out L | XLR | P – Audio Input Channel 1 |
| Ext Mon R | XLR | P – Audio Output Channel 2 |
| Ext Mon L | XLR | P – Audio Output Channel 1 |
| Mic/Line Input 4 | XLR | O – Audio Output 600 ohm Channel 2 |
| Mic/Line Input 3 | XLR | O – Audio Output 600 ohm Channel 1 |
| Mic/Line Input 2 | XLR | N – Audio Output 600 ohm Channel 1 |
| Mic/Line Input 1 | XLR | N – Audio Output 600 ohm Channel 2 |
| Editor | DB-15 | C – Audio Mixer 15P |
| Monitor L (up) | phono plug | R – Input to left channel |
| Monitor R (down) | phono plug | R – Input to right channel |

Table 2: Connections for Component A

## Component B – Equalizer

Component B is not connected to any other system component.

## Component C – Automatic Editing Control Unit

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| RS-232C | Blue Sony #4 – 40 UNC VCD Cable | I – RS-232C |
| VDU Out | BNC, black coax | J – Video In |
| Ext Sync In | BNC, red coax | Q – Black Burst Out 3 |
| Keyboard | 8-pin, black | K – (unlabeled) |
| Recorder | DB-9, black | P – Remote 1 (9P) |
| VTR C | DB-9, black | O – Remote 1 (9P) |
| Video Sw'er (25P) | DB-25, black | Q – RS-232C |
| Audio Mixer (15P) | DB-15, black | A – DB-15 |
| VTR | DB-9, black | N – Remote 1 (9P) |

Table 3: Connections for Component C

## Component D - Monitor

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video In | BNC, black coax | F – Video A Out |

Table 4: Connections for Component D

## Component E - Monitor

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Ext Sync In | BNC, blue coax | F – Ext Sync Out |

Table 5: Connections for Component E

## Component F – Monitor 1 #3

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Ext Sync Out | BNC, blue coax | E – Ext Sync In |
| Ext Sync In | BNC, blue coax | G – Ext Sync Out |
| Video A Out | BNC, black coax | D – Video In |
| Video A In | BNC, red coax | N – Video Out 2 (comp/non-comp) |

Table 6: Connections for Component F

## Component G – Monitor 2 #2

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video B In | BNC, blue coax | O – Video Out 2 |
| Ext Sync In | BNC, blue coax | H – Ext Sync Out |
| Ext Sync Out | BNC, blue coax | F – Ext Sync In |

Table 7: Connections for Component G

## Component H – Monitor Rec #1

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video B In | BNC, red coax | L – Video B Out |
| Ext Sync In | BNC, red coax | L – Ext Sync Out |
| Ext Sync Out | BNC, blue coax | G – Ext Sync In |

Table 8: Connections for Component H

## Component I - Microcomputer

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video Out | phono, black coax | not connected (other end of coax is BNC) |
| RS-232C | DB-25 | C – RS-232C |

Table 9: Connections for Component I

## Component J – Character Display

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video In | BNC, black coax | C – VDU Out |

Table 10: Connections for Component J

## Component K - Keyboard

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| unmarked |  | C – keyboard |

Table 11: Connections for Component K

## Component L – Monitor #4

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video A In | BNC, black coax | Q – Black Burst 4 |
| Video A Out | BNC, red coax | Q – Pan Out 3 |
| Video B In | BNC, blue coax | P – Video Output 2 |
| Video B Out | BNC, red coax | H – Video B In |
| Ext Sync In | BNC, blue coax | M – Sync Out |
| Ext Sync Out | BNC, red coax | H – Ext Sync In |

Table 12: Connections for Component L

## Component M - Monitor

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Video A Out | BNC, red coax | Q – Pst Out 1 |
| Sync In | BNC, green coax | disconnected (was connected to C?) |
| Sync Out | BNC, blue coax | L – Ext Sync In |

Table 13: Connections for Component M

## Component N – Videotape Player

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Ref | BNC, black coax | P – Video Input (on lower left) |
| Video In | BNC, gray coax | O – Ref |
| Video Out 1 | BNC, blue coax | Q – Video In 1 |
| Video Out 2 | BNC, red coax | F – Video A In |
| Audio Out Ch 1 | XLR | A – Mic/Line Input Ch 1 |
| Audio Out Ch 2 | XLR | A – Mic/Line Input Ch 2 |
| Remote 1 (9 pin) | DB-9, gray black | C – VTR |

Table 14: Connections for Component N

## Component O – Videotape Player

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Ref | BNC, gray coax | N – Video In |
| Video In | BNC, blue coax | Q – Gen Lock In |
| Video Out 1 | BNC, black coax | disconnected |
| Video Out 2 | BNC, blue coax | G – Video B In |
| Audio Out Ch 1 | XLR, gray | A – Mic/Line Input Ch 3 |
| Audio Out Ch 2 | XLR, gray | A – Mic/Line Input Ch 4 |
| Remote 1 (9 pin) | DB-9, black | C – VTR C |

Table 15: Connections for Component O

## Component P – Videotape Recorder

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Remote 1 (9P) | DB-9, black | C – Recorder |
| Video Input (upper left) | BNC, red coax | Wall Jack B |
| Video Input (lower left) | BNC, black coax | N – Ref |
| Video Input (right) | BNC, orange coax | Q – Pgm Out 1 |
| Video Output 1 | BNC, black coax | Q – Video In 6 |
| Video Output 2 | BNC, blue coax | L – Video B In |
| Audio Input Ch 1 | XLR | A – Line Out R |
| Audio Input Ch 2 | XLR | A – Line Out L |
| Audio Output Ch 1 | XLR | A – Ext Mon R |
| Audio Output Ch 2 | XLR | A – Ext Mon L |

Table 16: Connections for Component P

## Component Q – Special Effects Generator / Video Switcher

| Port Name | Connector and Wire | Destination (Component Code, Port) |
| --- | --- | --- |
| Gen Lock In | BNC, blue coax | O – Video In |
| RS-232C | DB-25, gray | C – Video Sw'er (25P) |
| Video In 1 | BNC, blue coax | N – Video Out 1 |
| Video In 4 | BNC, blue coax | Q – Gen Lock Out |
| Video In 6 | BNC, blue coax | P – Video Output 1 |
| Gen Lock Out | BNC, blue coax | Q – Video In 4 |
| Video Out 1 | BNC, blue coax | disconnected |
| Black Burst 3 | BNC, red coax | C – Ext Sync In |
| Black Burst 4 | BNC, black coax | L – Video A In |
| Pst Out 1 | BNC, red coax | M – Video A Out |
| Pgm Out 1 | BNC, orange coax | P – Video Input (right) |
| Pgm Out 3 | BNC, red coax | L – Video A Out |

Table 17: Connections for Component Q

Component R – Pair of Speakers – speaker inputs connect to Component A, Monitor L/R jacks